BayTech outlet # N

Roof Motor Control Box

Roof motor wall switch

5 VDC input

WebRelay

Relay Contacts

Input

115 VAC

Output

5 VDC

5 watts

+      -

Long lever snap switch

park position sensor

Up = Open

Down = Closed

Hi Sandy,

I have our 5 VDC power supply connected to the BayTech outlet #8.  This outlet also powers the roof motor.  The WebRelay and wall switch are in parallel and the park position sensor is in series.  In Observatory 5 there are, of course, two park position sensors in series.

Best,

Greg

Cat 5e cable to network switch